Architecture 
Three-tier hierarchical model (Access → Distribution/Core → Server/Security)

Topology 
Hierarchical star with 802.1Q trunked uplinks, /30 point-to-point routed links, and a firewall DMZ

| DEVICE | Rol | DESCRIPTION |
| --- | --- | --- |
| MS1 | Distribution / Core L3 | Router inter-VLAN, STP root, link to firewall |
| MS0 | Aggregation / Core L2 | Trunk link to MS1 and Access switches |
| MS2 | Gateway DMZ | Routing to servers, linked to firewall by DMZ |
| MS4, MS6, MS3, MS7 | Access | Switches via VLAN |
| Firewall2 | Internal perimeter | ASA between LAN and DMZ |

MS0 as an aggregation switch linked to MS4, MS6 and MS7 through trunks, also to MS1.

VLAN Design
LAB1, LAB2, ADMIN, ACADEMIC, SERVER

| VLAN | name | network | Gateway | Access Switch |
| --- | --- | --- | --- | --- |
| 10 | LAB1 | 192.168.10.0/24 | 192.168.10.1 (MS1) | MS4 |
| 20 | LAB2 | 192.168.20.0/24 | 192.168.20.1 (MS1) | MS6 |
| 30 | ADMIN | 192.168.30.0/24 | 192.168.30.1 (MS1) | MS3 |
| 40 | ACADEMIC | 192.168.40.0/24 | 192.168.40.1 (MS1) | MS7 |
| 50 | SERVER (DMZ) | 192.168.50.0/24 | 192.168.50.1 (MS2) | MS2 |

Switching
Trunking, access port config, and STP with MS1 as root bridge (priority 4096)

Security

Inside (security-level 100): Inner LAN Network

DMZ (security-level 50): Servers Network

ACLs applied

- INSIDE_IN

Allows traffic between 192.168.0.0/16 and 192.168.50.0/24

Only the LAN can initiate connections to the servers

Servers cannot initiate connections to the LAN

- ICMP_ALLOW

Allows echo, echo-reply, and unreachable connections within the INSIDE and DMZ

Facilitates diagnostics without opening arbitrary traffic

Inspection Policy

- inspect icmp in global_policy

the firewall tracks ICMP sessions using stateful inspection.

## Protocols summary

| Technology | detail |
| --- | --- |
| 802.1Q (dot1q) | Trunk encapsulation in all uplink links |
| Inter-VLAN Routing | MS1 and MS2 – ip routing |
| Spanning Tree (STP) | MS1 as a root bridge priority 4096 for VLANs 10, 20, 30, 40 |
| Static Routing | Static route MS1, MS2 and Firewall2 |
| Cisco ASA | Firewall nameif, security-level, policy-map y access-group |
| Switchport trunk/access | Segmentation L2 by area MSX |

## IP Addressing Table

MS0, MS3, MS4, MS6, and MS7 operate exclusively at Layer 2 — they have no L3 interfaces configured and therefore hold no IP addresses in the routed topology.

| Device | INTERFACE | IP Address | Subnet Mask | CIDR | Gateway | VLAN / Role |
| --- | --- | --- | --- | --- | --- | --- |
| MS1 | VLAN 10 SVI | 192.168.10.1 | 255.255.255.0 | /24 | — | LAB1 Gateway |
| MS1 | VLAN 20 SVI | 192.168.20.1 | 255.255.255.0 | /24 | — | LAB2 Gateway |
| MS1 | VLAN 30 SVI | 192.168.30.1 | 255.255.255.0 | /24 | — | ADMIN Gateway |
| MS1 | VLAN 40 SVI | 192.168.40.1 | 255.255.255.0 | /24 | — | ACADEMIC Gateway |
| MS1 | Gi1/0/3 (L3) | 10.0.0.1 | 255.255.255.252 | /30 | — | P2P → Firewall2 inside |
| Firewall2 | Gi1/1 (inside) | 10.0.0.2 | 255.255.255.252 | /30 | — | P2P → MS1 |
| Firewall2 | Gi1/2 (dmz) | 10.0.0.5 | 255.255.255.252 | /30 | — | P2P → MS2 |
| MS2 | Gi0/1 (L3) | 10.0.0.6 | 255.255.255.252 | /30 | 10.0.0.5 | P2P → Firewall2 dmz |
| MS2 | VLAN 50 SVI | 192.168.50.1 | 255.255.255.0 | /24 | — | SERVER Gateway |
| Hosts VLAN 10 | NIC | 192.168.10.2–254 | 255.255.255.0 | /24 | 192.168.10.1 | LAB1 clients |
| Hosts VLAN 20 | NIC | 192.168.20.2–254 | 255.255.255.0 | /24 | 192.168.20.1 | LAB2 clients |
| Hosts VLAN 30 | NIC | 192.168.30.2–254 | 255.255.255.0 | /24 | 192.168.30.1 | ADMIN clients |
| Hosts VLAN 40 | NIC | 192.168.40.2–254 | 255.255.255.0 | /24 | 192.168.40.1 | ACADEMIC clients |
| Servers VLAN 50 | NIC | 192.168.50.2–254 | 255.255.255.0 | /24 | 192.168.50.1 | DMZ servers |

### P2P Link Summary

| Link | Network | Usable Range | Device A | Device B |
| --- | --- | --- | --- | --- |
| MS1 ↔ Firewall2 | 10.0.0.0/30 | 10.0.0.1 – 10.0.0.2 | 10.0.0.1 (MS1 Gi1/0/3) | 10.0.0.2 (FW2 inside) |
| Firewall2 ↔ MS2 | 10.0.0.4/30 | 10.0.0.5 – 10.0.0.6 | 10.0.0.5 (FW2 dmz) | 10.0.0.6 (MS2 Gi0/1) |

#### Assessment Task 2: Checklist

- Develop a troubleshooting and monitoring plan to optimise the performance and reliability of an IP network?

- Develop a troubleshooting and monitoring plan that outlines how network segments will be isolated for troubleshooting procedures?.

- Develop a troubleshooting and monitoring plan that includes a plan for testing sequences and scenarios of the IP network infrastructure?

- Develop a troubleshooting and monitoring plan that identifies suitable network testing and monitoring tools, and software applications for troubleshooting and monitoring?.

- Develop a troubleshooting and monitoring plan that includes a routine internetwork operating system (IOS) device maintenance plan for the monitoring of routing protocols and router configurations?

- Develop a troubleshooting and monitoring plan that includes troubleshooting and monitoring of security issues related to IOS services for mission critical applications?

- Develop a troubleshooting and monitoring plan that includes a plan for troubleshooting and monitoring internet protocol version 6 (IPv6) and version 4 (IPv4) interoperability?

- Manage and monitor strategies for network monitoring structure?

- Isolate sub-optimal internetwork operations at open systems interconnection (OSI) model layer relevant to the RTO’s network?

- Test switch-to-switch connectivity for the IP network?

- Test access ports and loop prevention for the virtual local area network (VLAN)?

- Test private VLANs relevant to the IP network?

- Test switch virtual interfaces (SVI) relevant to the IP network?

- Test switch support of advanced services relevant to the IP network?

- Troubleshoot switch configuration relevant to the IP network?